DORDT COLLEGE

ENGINEERING DEPT._____________________Proakis & Salehi_	NAME___________________

COURSE___EGR 363                                    SUBJECT__PS#_____	PAGE____ OF____ DATE__________

3-A  An investigation of TV channel allocations and image frequency sensitivities.

a.)	Use https://www.fcc.gov/media/radio/find-community-coordinates   to find Sioux Center’s latitude and longitude.

b.)	Use https://www.fcc.gov/media/television/tv-query to create a list of all the TV stations within 100 km of Sioux Center.  Choose “TV List (intermediate detail, fast).”  When you enter the latitude and longitude you may omit letters N and W.  They will be assumed since all U.S. locations are so located.  A text size of “7” allowed me to view a whole row of data in the width of my screen.  The column headings are missing!  They should be, from left to right:  Call Sign, RF channel number, Service type, License status, City, State, Country, FCC file number, Facility ID number, Effective Radiated Power, Antenna Height Above Average Terrain (HAAT), Antenna Height Above Mean Sea Level (AMSL), Antenna Height Above Ground Level (AGL), Antennal N Latitude, Antenna Longitude, Antenna Registration Number (link to a bunch of data about the antenna), Distance from Sioux Center in km, Distance from Sioux Center in Statute miles, Azimuth (compass bearing from Sioux Center to the antenna), Owner.

c.)	In the list of TV stations, find KSIN–TV (The “DT” means this is the currently licensed digital transmitter for station KSIN in Sioux City.  Other descriptors are “LD” for “Low power digital,” “TX” stands for “translator” which may be an analog repeater, “CP” stands for “construction permit,” but the final license has not yet been granted and “APP” stands for “pending application.”)  If you click on the call letters you will get a detailed report on the transmitter.  Most of the numbers in the detail appear in the columns of the summary report, but the summary report often omits the headings for the columns!  What is the effective radiated power (ERP) and height above average terrain (HAAT) for the antenna used by station KSIN–TV–DT–LIC?  Repeat for KMEG–DT–LIC

d.)	Digital TV stations are assigned a certain channel number by the FCC and must broadcast on that channel.  They are also allowed to broadcast a digital code that identifies their channel number. This broadcast code is called their virtual channel number.  The FCC does not require that the virtual channel number match the assigned transmitter (or RF) channel number.  KSIN–DT–LIC happens to transmit a code for virtual channel 27 (Actually 27.1,  27.2 and 27.3 since a digital TV station can broadcast more than one program at a time should it choose to do so.  This station transmits three programs from PBS or IPTV simultaneously.)  Thus to viewers, KSIN appears to be “Channel 27.”  What is the assigned channel number for KSIN–DT–LIC’s transmission?  (The “Transmit Channel” data in the FCC list.)  Repeat for KMEG–DT–LIC.  KMEG reports a virtual channel number of “14” to viewers.

Note:  The assigned channel is the actual channel that a digital television station may transmit on.  It only reports a “virtual channel number” to viewers for marketing reasons.  (The old analog KSIN TV station was really on channel 27—nothing virtual about that.  The analog transmitters were turned off between February and June of 2009.  Stations did not want to have to teach their viewers new channel numbers, thus virtual channel numbers were invented.)  Digital television receivers must “scan for channels.”  This fills up a lookup table.  When the viewer asks to watch (virtual) channel 14 for example, the TV set looks at address 14 in the table to find the actual assigned channel for that station, then tunes in to the actual assigned channel.  Usually viewers are completely unaware that this translation happens.

e.)	Use Wikipedia to look up “North American broadcast television frequencies.”  What is the carrier frequency (also called the “ATSC pilot” frequency) for KSIN–DT–LIC?  Also, how wide is the channel allocation they have (in MHz, upper edge to lower edge)?  (The Video and audio carrier frequencies apply only to analog NTSC type transmitters.  Digital TV transmitters no longer use “video” or “audio” carriers, just ignore those columns.)

f.)	Based on the carrier frequency’s location within the assigned frequency band, is this DSB-SC, VSB, or SSB modulation?  Note that an SSB signal would have no need for any spectrum on the unused side of the carrier.

g.)	What is the image frequency for KSIN–DT–LIC?  The FCC supported intermediate frequency for (analog and digital) TV receivers is 45.75 MHz with high-side tuning.  The local oscillator runs at a frequency greater than the carrier frequency.  What actual channel number corresponds best with this image frequency?  Return to the FCC list of TV stations.  Is anything assigned on the image frequency?  If so, what?  Who owns the assignment(s) on the image channel and how does this compare with the ownership of KSIN–DT–LIC?  Why does this situation work?